RUBRIC    INSTRUCTIONS

| Research Reference | Team/Name Paintbrush 2 | Reference #1 |
| --- | --- | --- |

Research Area (bold one that best fits):

Exploration of existing solutions      Governs the problem    Background	  Business perspective

Source Type 		Peer-reviewed: 	Book    	Journal		Other (patent)

(bold one)  		Non-peer-reviewed: 	Government	Web post	Other

Citation (include enough so source can be found easily; use MLA, APA, or IEEE citation style)

BASF SE. (2020). Cleaning composition (European Patent No. EP3589233B1). European Patent Office. Retrieved from https://patents.google.com/patent/EP3589233B1/en

Summary of key points from source

- The patent introduces a cleaning composition that uses silicone-based ingredients along with surfactants and other functional additives to remove dirt, residues, grease, or coating agents. - Silicone compounds are highlighted for their ability to penetrate and break down oily or resinous deposits while leaving behind a surface that resists re-soiling. - Compared to traditional strong organic solvents (like toluene or xylene), these silicone-based cleaners are designed to be less volatile, thereby reducing contribution to smog and indoor air quality problems. - Environmentally, silicones tend to be less acutely toxic and offer lower VOC emissions, making them a more sustainable option. However, persistence in the environment can vary based on chain length and structure, meaning degradation rates differ for different silicone compounds. - From a cleaning-performance perspective, silicone-based formulations provide excellent solubilization power against hydrophobic residues such as paints, varnishes, and adhesives, while maintaining material compatibility (less damaging to bristles, plastics, or coated surfaces). - The composition can be modified with non-ionic and anionic surfactants to improve the removal of both oily and water-based soils, making the formulation versatile for different industrial and consumer cleaning tasks. - Overall, the patent claims an improvement in cleaning efficiency, sustainability, and safety when compared to conventional solvent-heavy mixtures.

| Research Reference | Team/Name Paintbrush 2 | Reference #2 |
| --- | --- | --- |

Research Area (bold one that best fits):

Exploration of existing solutions      Governs the problem    Background	  Business perspective

Source Type 		Peer-reviewed: 	Book    	Journal		Other

(bold one)  		Non-peer-reviewed: 	Government	Web post	Other

"Ryobi paintbrush cleaner." The Family Handyman, vol. 60, no. 5, May 2010, p. 8. Gale General OneFile, link.gale.com/apps/doc/A225786616/ITOF?u=duke_perkins&sid=summon&xid=033b0258. Accessed 11 Sept. 2025. https://go-gale-com.proxy.lib.duke.edu/ps/i.do?p=ITOF&u=duke_perkins&id=GALE%7CA225786616&v=2.1&it=r&sid=summon&aty=ip

Summary of key points from source

- $30 paintbrush cleaner - Stick a dirty paintbrush in, turn it on, and for 1 minute it swipes the brush back and forth - “Stiff, counterrotating brushes scrub both sides at once” - Cleaning the machine is easy - just snap off the lid and rinse it out - Issue: cant use flammable solvent because electric motors and flammable odors don’t go well together. So, you have to use soy-based solvents - model No. BC400 at ryobitools.com

| Research Reference | Team/Name Paintbrush 2 | Reference #3 |
| --- | --- | --- |

Research Area (bold one that best fits):

Exploration of existing solutions      Governs the problem    Background	  Business perspective

Source Type 		Peer-reviewed: 	Book    	Journal		Other

(bold one)  		Non-peer-reviewed: 	Government	Web post	Other

Citation (include enough so source can be found easily; use MLA, APA, or IEEE citation style)

Watson, Greg. “Cleaning Brushes without Solvents ¦ Just Paint.” Just Paint, 13 Sept. 2019, justpaint.org/cleaning-brushes-without-solvents/. Accessed 14 Sept. 2025.

Summary of key points from source

We are also considering the alternative of not using solvents to clean the paintbrushes but rather oil itself. This source gives a detailed explanation, step by step, of the general process of cleaning paint brushes using drying oils. The source talks about: - Using drying oils for intermediate cleaning - Step by step process - Final clean with soap and water - Advantages and some considerations - Choosing between drying vs non-drying oils - Waste management

| Research Reference | Team/Name Paintbrush 2 | Reference #4 |
| --- | --- | --- |

Research Area (bold one that best fits):

Exploration of existing solutions      Governs the problem    Background	  Business perspective

Source Type 		Peer-reviewed: 	Book    	Journal		Other

(bold one)  		Non-peer-reviewed: 	Government	Web post	Other

Citation (include enough so source can be found easily; use MLA, APA, or IEEE citation style)

Evie Hatch Evie’s interests lie in the history, et al. “Cleaning Fine Art Painting Brushes.” Jackson’s Art Blog, 20 July 2021, www.jacksonsart.com/blog/2021/07/20/brush-cleaning-guide-for-all-mediums/.

Summary of key points from source

- Cleaning methods vary depending on oil, acrylic, or watercolor paint. - Differentiates for when you would need to use solvents (goes into which types), when you would use soap and water, or when you would just use soap - Use brush cleaners and soaps to safely remove paint and maintain bristle quality. - Follow specific steps for each medium to avoid damage and extend brush life. - Explain which strategies apply for which brush type and how to apply them - Always reshape brushes after washing and dry them flat to preserve their form.

| Research Reference | Team/Name Paintbrush 2 | Reference #5 |
| --- | --- | --- |

Research Area (bold one that best fits):

Exploration of existing solutions      Governs the problem    Background	  Business perspective

Source Type 		Peer-reviewed: 	Book    	Journal		Other

(bold one)  		Non-peer-reviewed: 	Government	Web post	Other

Citation (include enough so source can be found easily; use MLA, APA, or IEEE citation style)

Evolve Artist Staff. “How to Clean Oil Paint Brushes in 4 Easy Steps.” Evolve Artist, 28 Nov. 2018, evolveartist.com/blog/how-to-clean-oil-paint-brushes/. Accessed 15 Sept. 2025.

Summary of key points from source

This article presents an alternative method of cleaning a paintbrush that doesn’t involve a solvent but instead uses a brick of soap to rub the paintbrush on. The method works quite simply by getting a bar of soap, then wetting it by holding it under a faucet, and then moving the paintbrush back and forth across the flat of the paintbrush rather than pressing down with the tip. The paintbrush is then rinsed and the process is repeated until there is no colored residue left over on the soap bar after rubbing the brush back and forth.

| Research Reference | Team/Name Paintbrush 2 | Reference #6 |
| --- | --- | --- |

Research Area (bold one that best fits):

Exploration of existing solutions      Governs the problem    Background	  Business perspective

Source Type 		Peer-reviewed: 	Book    	Journal		Other

(bold one)  		Non-peer-reviewed: 	Government	Web post	Other

Omegasonics. Can Ultrasonic Cleaners Be Used to Clean Paintbrushes? Knowledge-Center, Omegasonics, https://www.omegasonics.com/knowledge-center/blog/can-ultrasonic-cleaners-be-used-to-clean-paintbrushes/ Accessed 10 Sept. 2025

Summary of key points from source

- Ultrasonic cleaners use sound waves to generate microscopic cavitation bubbles. These bubbles agitate the liquid and items submerged in the bath in order to clean the items. - Cleaners are currently marketed for “deep clean” or “restoration” of brushes, especially those with paint stuck deep in bristles by workshop workers. - They can clean brushes in under 10 minutes depending on the paint type and hardness. - Can be more effective if combined with other cleaning solutions. - Reaches parts of the brush that manual scrubbing struggles to, bristles receive less mechanical wear. - May be dangerous by building up heat or evaporating toxic solvents.

| Research Reference | Team/Name Paintbrush 2 | Reference #7 |
| --- | --- | --- |

Research Area (bold one that best fits):

Exploration of existing solutions      Governs the problem    Background	  Business perspective

Source Type 		Peer-reviewed: 	Book    	Journal		Other

(bold one)  		Non-peer-reviewed: 	Government	Web post	Other(Product listing)

Citation (include enough so source can be found easily; use MLA, APA, or IEEE citation style)

YAGHVEO® Electric Paint Brush Cleaner Rinse Cup. Amazon.com, YAGHVEO, https://www.amazon.com/YAGHVEO-Electric-Improvement-Automatic-Watercolor/dp/B0BXNGZ1RY . Accessed 10 Sept. 2025

Summary of key points from source

- An electric rinse cup containing a rotating spinner. - Holds 1-8 brushes at once. - Various silicone cleaning heads to contact brushes to clean bristles more uniformly. - USB-C chargeable and portable for use in studios, home or classroom. - Designed for acrylic, watercolor, oil and tempera. - Very fast cleaning in under a minute for some brushes, also helps to dry brushes by spinning. - May wear down delicate bristles and may not reach deeply embedded or dried up paint.

| Research Reference | Team/Name Paintbrush 2 | Reference #8 |
| --- | --- | --- |

Research Area (bold one that best fits):

Exploration of existing solutions      Governs the problem    Background	  Business perspective

Source Type 		Peer-reviewed: 	Book    	Journal		Other (patent)

(bold one)  		Non-peer-reviewed: 	Government	Web post	Other

Citation (include enough so source can be found easily; use MLA, APA, or IEEE citation style)

"Self cleaning paint brush." Long Island Business News, vol. 48, no. 16, 20 Apr. 2001, p. 42A. Gale General OneFile, link.gale.com/apps/doc/A74364977/ITOF?u=duke_perkins&sid=summon&xid=6fcdf383. Accessed 9 Sept. 2025. https://go-gale-com.proxy.lib.duke.edu/ps/i.do?p=ITOF&u=duke_perkins&id=GALE%7CA74364977&v=2.1&it=r&sid=summon&aty=ip

Summary of key points from source

- In 2000, inventor, Raphael Isaac, created a self cleaning paintbrush - His design involved a hollow paintbrush handle that brings water through it and to the bristles - Diverting bars control the flow of water through the bristles - A patent exists on this idea

| Research Reference | Team/Name Paintbrush 2 | Reference #9 |
| --- | --- | --- |

Research Area (bold one that best fits):

Exploration of existing solutions      Governs the problem    Background	  Business perspective

Source Type 		Peer-reviewed: 	Book    	Journal		Other (patent)

(bold one)  		Non-peer-reviewed: 	Government	Web post	Other

Citation (include enough so source can be found easily; use MLA, APA, or IEEE citation style)

Winters, J., & Winters, D. (2006). Paint brush cleaner (U.S. Patent Application No. US20060213544A1). U.S. Patent and Trademark Office. Retrieved from https://patents.google.com/patent/US20060213544A1/en

Summary of key points from source

A paintbrush cleaning device designed to remove paint effectively from bristles. It uses a container with cleaning fluid and has a system that applies pressurized fluid spray or jets to clean bristles thoroughly. Includes variations for both manual and powered embodiments (e.g., electrically powered pump). Shooting water streams into a closed compartment where the brush resides. It situates itself as an improvement over prior devices, making paintbrush cleaning more efficient, economical, and environmentally friendly.

| Research Reference | Team/Name Paintbrush 2 | Reference #10 |
| --- | --- | --- |

Research Area (bold one that best fits):

Exploration of existing solutions      Governs the problem    Background	  Business perspective

Source Type 		Peer-reviewed: 	Book    	Journal		Other

(bold one)  		Non-peer-reviewed: 	Government	Web post	Other

“Painting Tools Market Size, Statistics ¦ Industry Trends Report 2028.” Global Market Insights Inc., www.gminsights.com/industry-analysis/painting-tools-market.

Summary of key points from source

This source discusses a potential solution when considering a business perspective. It provides statistics on the current market for paint brushes and subsequent cleaning solutions. It shows projections for the upcoming years and can give us insight into what type of product consumers may be looking for. It discusses factors that drive demands which can align with drives for our product. It also talks about sustainability trends, which while not at the top of the list of importance for us, should definitely be accounted for.

| Research Reference | Team/Name Paintbrush 2 | Reference #11 |
| --- | --- | --- |

Research Area (bold one that best fits):

Exploration of existing solutions      Governs the problem    Background	  Business perspective

Source Type 		Peer-reviewed: 	Book    	Journal		Other

(bold one)  		Non-peer-reviewed: 	Government	Web post	Other

Tobiszewski, M., & Bystrzanowska, M. (2020). Monetary value estimates of solvent emissions. Green Chemistry, 22(22), 7879–7892. https://doi.org/10.1039/D0GC03210G

Summary of key points from source

The study quantifies the environmental costs of solvent emissions by converting their impacts into monetary values. Evaluated 52 solvents across 13 air-impact categories (climate and health related) and two water-impact categories, using a multi-compartment distribution model (air, water, soil, sediment). Impact values ranged $0.70–1179.51 per liter. Low impact solvents: medium-chain aliphatic hydrocarbons (pentane, hexane, heptane), alcohols, ethers,acetone, MEK, esters, and organic acids (acetic, lactic). High impact solvents: long-chain aliphatic hydrocarbons (decane, dodecane), aromatic hydrocarbons (benzene, toluene, xylenes), limonene, α-pinene, β-pinene, chloroform, trichloroethene, and carbon tetrachloride. Environmental fate: Nonpolar solvents (hydrocarbons, terpenes, chlorinated solvents) persist in air. Polar solvents (alcohols, organic acids) are mostly found in water. Aldehydes, ketones, and esters partition between both phases.

| Research Reference | Team/Name Paintbrush 2 | Reference #12 |
| --- | --- | --- |

Research Area (bold one that best fits):

Exploration of existing solutions      Governs the problem    Background	  Business perspective

Source Type 		Peer-reviewed: 	Book    	Journal		Other

(bold one)  		Non-peer-reviewed: 	Government	Web post	Other

Citation (include enough so source can be found easily; use MLA, APA, or IEEE citation style)

Gandhi, Anika. “How to Clean Dried Paint Brushes: Testing 7 Methods.” Anika’s DIY Life, 27 Sept. 2024, www.anikasdiylife.com/how-to-clean-dried-paint-brushes/.

Summary of key points from source

This source compares different solvents at cleaning a large paintbrush with paint stuck in the bristles. The source compares the following solvents: Warm vinegar, Goo Gone, Rubbing Alcohol (with a plastic wrap on top to prevent evaporation), Simple Green, Laundry Detergent and water mix (1:3), Dawn Dish Soap and water (1:3), Murphy’s Oil Soap. After 3 hours of soaking in the solvent the following qualitative analysis was observed: Warm Vinegar – Pretty much like it was at the 90-minute mark, Goo Gone – No change at all, Rubbing Alcohol – There are a lot of paint particles in the liquid, and the bristles feel really soft, Simple Green – Starting to soften up a little bit, Laundry Detergent and water mix (1:3)- It seems to be getting slightly softer, Dawn Dish Soap and water (1:3) – Not much different from the 90-minute mark, Murphy’s Oil Soap – Still the same. When rinsed with water, the one soaked in rubbing alcohol for 3 hours had no paint on it, while after 16 hours after rinsed with water they all contained a large amount of paint left over. See the image below for more details:

| Research Reference | Team/Name Paintbrush 2 | Reference #13 |
| --- | --- | --- |

Research Area (bold one that best fits):

Exploration of existing solutions      Governs the problem    Background	  Business perspective

Source Type 		Peer-reviewed: 	Book    	Journal		Other

(bold one)  		Non-peer-reviewed: 	Government	Web post	Other

admin. “Chemical Paint Strippers: Everything You Need to Know.” CRC Industries - NZ, 2023, www.crc.co.nz/blog/chemical-paint-strippers-everything-you-need-to-know/. Accessed 15 Sept. 2025.

Summary of key points from source

This post details the chemistry of how paint strippers work—this article was useful in understanding what properties of solvents should be looked at to create the most effective paint brush cleaning machine. Paint strippers work by breaking down the chemical bonds of the paint, which is often done through “hydrolysis of the chain bonds of the polymers forming the paint.” Most active paint strippers contain dichloromethane which is often also called methylene chloride. Solvent paint strippers generally have a gel-like consistency (that seems to be to dense for the project we’re working on but it is something to consider). If a paint stripper includes methylene chloride, it can be toxic and a mask but be warn (our project should aim to have our device not have this substance in it, or at least in lower concentrations). It is true that the longer a paint stripper is applied to a surface, the longer it has to work—it seems to be a linear relationship with time. Paint strippers can destroy and break down materials like vinyl and plastics—either removing elements from the solvent that break these down is a good idea if using 3D printed parts, or making parts potentially out of metal.

| Research Reference | Team/Name Paintbrush 2 | Reference #14 |
| --- | --- | --- |

Research Area (bold one that best fits):

Exploration of existing solutions      Governs the problem    Background	  Business perspective

Source Type 		Peer-reviewed: 	Book    	Journal		Other

(bold one)  		Non-peer-reviewed: 	Government	Web post	Other: Magazine article

“Cleaning Paint Brushes the Right Way.” The Family Handyman, vol. 49, no. 5, May 1999, pp. 56–64. Home Service Publications, Inc. https://go-gale-com.proxy.lib.duke.edu/ps/i.do?p=ITOF&u=duke_perkins&id=GALE%7CA54588553&v=2.1&it=r&sid=summon&asid=c513a1a8

Summary of key points from source

- Bad habit: Some painters will delay cleaning by wrapping brushes in plastic and freezing them. This may cause odors to form and also contaminate other products in the freezer with paint chemicals. - A well maintained brush should last for years and dozens of uses if cleaned correctly. - High quality matters: $16 brushes will outperform $3 brushes. - Latex paints should be used with nylon or polyester brushes. - Oil paints should be used with natural brushes. - In order to clean latex/water-based paint: - Scrape off excess paint on a can edge - Wash in soapy water and work paint free with hands and brush comb. - Spin brush to remove paint. - Rinse out and repeat. - Wrap bristles in heavy paper to retain brush shape. - In order to clean oil-based paints or varnishes: - Rinse in paint thinner/solvent and work bristles with a gloved hand or comb. - Spin brush with a brush spinner. - Dip brush in fresh paint thinner and agitate for 2 min. - Spin again. - Quick wash in soapy water. - Clean outdoors or in well ventilated space away from flames or sparks. - Let solids settle in the solvent before pouring off solvent for reuse. - To revive “petrified brushes”; - Use a solvent based cleaner and soak over night. - Comb out softened paint and resoak if necessary. - Repeat previous cleaning steps. - Recommended tools: - Brush combs - Brush and roller spinner - Sav-a-Brush

| Research Reference | Team/Name Paintbrush 2 | Reference #15 |
| --- | --- | --- |

Research Area (bold one that best fits):

Exploration of existing solutions      Governs the problem    Background	  Business perspective

Source Type 		Peer-reviewed: 	Book    	Journal		Other

(bold one)  		Non-peer-reviewed: 	Government	Web post	Other: Magazine article

Cano-Murillo, Kathy. “The BEST Way to Clean Paint Brushes.” Crafty Chica, 23 Mar. 2017, craftychica.com/2017/03/correct-way-clean-paint-brushes/. Accessed 15 Sept. 2025. ‌

Summary of key points from source

This article/webpost outlines the best practices when cleaning brushes. This information will be useful when we need to know how to best create a mechanism that moves the brushes as to not damage them. Brushes, when cleaned, should never face tip down as that risks damaging the bristles and spreading them apart. The solvent and liquids that help break down the brush should not extend high onto the brush as that could lead to the adhesive that attaches the bristles to the wooden stem to wear. The article also recommends that brushes be run under cool water to remove any solvent or soap present. In addition, the article recommends to treat the brush hair like one’s own hair, not applying dish soap to the bristles but applying shampoo and/or conditioner to the bristles to wash paint away.

| Research Reference | Team/Name Paintbrush 2 | Reference #16 |
| --- | --- | --- |

Research Area (bold one that best fits):

Exploration of existing solutions      Governs the problem    Background	  Business perspective

Source Type 		Peer-reviewed: 	Book    	Journal		Other

(bold one)  		Non-peer-reviewed: 	Government	Web post	Other

Radtke, David. "Cleaning paint brushes." The Family Handyman, vol. 49, no. 5, May 1999, p. 56. Gale General OneFile, link.gale.com/apps/doc/A54588553/ITOF?u=duke_perkins&sid=summon&xid=c513a1a8. Accessed 10 Sept. 2025. https://go-gale-com.proxy.lib.duke.edu/ps/i.do?p=ITOF&u=duke_perkins&id=GALE%7CA54588553&v=2.1&it=r&sid=summon

Summary of key points from source

- Some brushes are filled with hard & dry paint - Solvent based cleaner is good for natural or synthetic brushes - Water-based cleaner is only good for synthetic brushes - Brushes should soak in solvent overnight in a covered container. If paint isn't softened after 24 hrs, let brushes soak another day - Ensure brushes are suspended so they don’t change shape into a “hockey stick” shape - Once soft, comb away solids - Then, wash like you would a normal brush - This paper recommends Sav-A-Brush

| Research Reference | Team/Name Paintbrush 2 | Reference #17 |
| --- | --- | --- |

Research Area (bold one that best fits):

Exploration of existing solutions      Governs the problem    Background	  Business perspective

Source Type 		Peer-reviewed: 	Book    	Journal		Other

(bold one)  		Non-peer-reviewed: 	Government	Web post	Other

Shin, H. M., Cathey, T., Maddalena, R., Wilson, N. K., & McKone, T. E. (2014). Evaluation of chemical exposures for use in epidemiological studies of painters. Journal of Applied Toxicology, 34(8), 868–879. https://doi.org/10.1002/jat.3979

Summary of key points from source

- Topic: chemical exposures experienced by painters.. Common solvents studied include toluene, xylene, ethylbenzene, and ketones (like methyl ethyl ketone, MEK). - Toluene – effective at dissolving many paint resins; contributes to smog formation (as a volatile organic compound, VOC); toxic to the central nervous system, potentially causing headaches, dizziness, and long-term neurological effects. - Xylene – another strong aromatic solvent, excellent for thinning oil-based paints and cleaning; environmentally, it persists as an air pollutant and contributes to ground-level ozone; health risks include respiratory irritation and possible liver/kidney impacts with chronic exposure. - Ethylbenzene – moderately strong solvent, often present in solvent mixtures; environmental concern due to its persistence in soil and groundwater; health effects include ear and auditory damage seen in animal studies, along with possible carcinogenic potential. - Ketones (e.g., MEK) – fast-evaporating solvents, very effective at weakening and dissolving paint binders; though less acutely toxic than aromatics, they contribute to atmospheric VOC load and can cause short-term irritation of eyes, nose, and throat. - The study concludes that while these solvents are valuable for dissolving and removing paint, their combined health hazards and environmental persistence justify efforts to develop improved exposure controls or alternatives.